Anna JSON Operational Policy
(Runtime Behavior, Provisioning, and Deployment Rules)

Version: v1.0

Status: Internal / Partner Reference

Authors: AnnaSoft Inc. Nemo & Anna Platform Team

Last Updated: 2026-03-20

License:
This document may be shared with partners and developers working with the Nemo & Anna platform.
Public redistribution may require permission from AnnaSoft Inc.

0. Purpose

This document defines the operational policy around Anna JSON deployment, firmware behavior, commissioning, reset behavior, certification path, and runtime update rules.

This document is not the field-level schema definition.
Field definitions belong to Anna JSON FunctionType Specification.

1. MVP Scope

For MVP:

- Matter certification assets such as DAC/PAI are not included in Anna JSON

- Official Matter certification flow is handled separately

- Certification bundle flashing will be supported later through a dedicated local tool

2. Hardware Policy

Reference board: ESP32-C6 DevKitC-1

- Input GPIO: 0, 1, 2, 3

- Output GPIO: 18, 19, 20, 21, 22, 23

Board guidance:

- left side = input

- right side = output

Any GPIO outside this range is treated as invalid by both server and firmware policy.

3. Layer Separation Policy

3-1. Firmware policy layer

Defined outside Anna JSON:

- DataModelRevision

- SpecificationVersion

- CapabilityMinima

- MaxPathsPerInvoke

- CommissioningFlow

- DiscoveryCapabilities

- FirmwareHash

3-2. Product layer

Managed in ProductInfo:

- VID / PID

- vendor / product naming

- HW/SW versions

- user-visible labels

- certification status

3-3. Unit layer

Managed in UnitInfo:

- SerialNumber

- UniqueID

- Setup Discriminator

- Setup PIN / Verifier

3-4. Action layer

Managed in Anna JSON action definitions:

- Button

- Switch

- Modes

- ConButton

- ConSwitch

3-5. Implementation Limits
The current reference implementation supports:
• Up to 5 Button / ConButton instances
• Up to 3 Switch / ConSwitch instances
• Up to 6 total action instances per device
• One Modes set per device

4. Identifier Policy

4-1. SerialNumber

- External-facing identifier

- Example format: YYMMDD + XXXXXX

- Example: 250717000001

4-2. UniqueID

- Internal-only unique identifier

- Uses ULID-based format

- Must not be exposed in QR or user-facing UI

- May be stored in device NVS and server DB

4-3. UserID

User account identifiers are outside Anna JSON specification and belong to platform/server policy.

5. Commissioning Policy

The following are device-level provisioning data, not Anna JSON schema fields:

- Setup PIN Code

- Discriminator

- SPAKE2+ verifier

- QR data

Current policy

- SetupPINCode / Discriminator use ESP-Matter HRNG auto generation

- Policy may be:

  - fixed: preserved after Factory Reset

  - dynamic: regenerated after Factory Reset and saved again

6. Factory Reset Policy

Trigger

- Power ON/OFF repeated 5 times within approximately 10 seconds

- ScheduleFactoryReset() policy target

- Current implementation status may differ by firmware branch

Effects

Factory Reset clears runtime and app-managed state, but does not automatically redefine Anna JSON itself.

Cleared items

- app-owned NVS data

- user preferences

- network credentials

- runtime state

- commissioning / fabric state

Preserved or regenerated

- firmware remains

- SerialNumber remains

- UniqueID remains

- Setup PIN / Discriminator follow fixed/dynamic policy

7. Apply Validation Policy

Before applying a new Anna JSON payload, the system must validate:

- output pin conflicts

- protection GPIO rules

- mode consistency

- allowed GPIO range

- payload integrity

Invalid payloads must be rejected before runtime application.

8. Mode Runtime Policy

- Modes are implemented using runtime/cluster policy, not as free-floating labels

- Only one mode may be active at a time

- Last active mode is stored in NVS

- On reboot, stored mode is restored

- If stored mode is invalid or blocked by NotModePin, system falls back to mode index 0

- If a Mode Off operation occurs, system returns to mode index 0

9. ON/OFF State Recovery Policy

StartUpOnOff recovery uses Matter StartUpOnOff = 2 as the default basis.

However, the final restored state must also consider:

- protection GPIO conditions

- stored state integrity

- initialization safety policy

If those checks fail, the final state may be corrected to safe OFF.

10. Update Path Policy

Development / Pre-certification devices

Runtime update paths may be allowed:

- local IP / HTTPS

- vendor cluster path

- OTA-assisted config delivery

- pull after push notification

Certified release devices

Certified release devices should restrict configuration changes to trusted local flashing procedures only.

Runtime server-side configuration injection should be disabled in certified release mode.

11. Certification Policy

For release devices:

- CSA-issued VID/PID

- official DAC/PAI

- CertificationID recorded

- certification bundle applied through trusted local tool

- HSM-linked manufacturing tool may be used

Anna JSON itself remains an action-definition layer and does not carry certification assets.

12. Firmware Constant Policy

Firmware constants for the current reference profile:

- DataModelRevision = 1

- SpecificationVersion = 0x01040100

- CapabilityMinima: SubscriptionsPerClient=4, Fabrics=3

- MaxPathsPerInvoke = 2

- CommissioningFlow = 0

- DiscoveryCapabilities = 0x03

- FirmwareHash: generated from firmware.bin SHA-256 and injected into read-only firmware constants

FirmwareHash applies to firmware integrity, not to mutable ProductInfo or Anna JSON content.

13. Release Separation Policy

A certified release device should satisfy release conditions as a set, not through a single flag only.

Recommended consistency checks:

- release VID/PID

- official DAC/PAI

- release firmware profile

- release signing chain

If any of these are inconsistent, release path should be rejected or fallback to safe handling.

14. Document Boundary

This operational document includes

- commissioning policy

- reset policy

- update path policy

- firmware constant policy

- certification separation policy

- runtime validation rules

This operational document excludes

- field-by-field Anna JSON schema

- FunctionType field definitions

- per-field type tables

Those belong to:

Anna JSON FunctionType Specification

Language Notice

English is the normative version of this document.
The Korean text below is provided for internal reference and documentation management.

If there is any discrepancy, the English version prevails.

언어 공지

이 문서의 공식 기준 언어는 영어입니다.
아래의 한국어 번역은 내부 참고 및 문서 관리를 위해 제공됩니다.

해석 차이가 있을 경우 영어 버전을 기준으로 합니다.

Anna JSON 운영 정책

(Runtime Behavior, Provisioning, Deployment Rules)

Version: v1.0
Status: Internal / Partner Reference
Authors: AnnaSoft Inc. Nemo & Anna Platform Team
Last Updated: 2026-03-07

License
이 문서는 Nemo & Anna 플랫폼과 함께 작업하는 파트너 및 개발자와 공유될 수 있습니다.
단, 공개 재배포는 AnnaSoft Inc.의 허가가 필요할 수 있습니다.

0. 목적 (Purpose)

이 문서는 Anna JSON 배포, 펌웨어 동작, 커미셔닝, 리셋 동작, 인증 경로, 런타임 업데이트 규칙과 관련된 운영 정책을 정의합니다.

이 문서는 필드 수준의 스키마 정의 문서가 아닙니다.

각 필드 정의는 다음 문서에 속합니다.

Anna JSON FunctionType Specification

1. MVP 범위 (MVP Scope)

MVP 단계에서는 다음 정책을 따릅니다.

- DAC / PAI 와 같은 Matter 인증 자산은 Anna JSON에 포함되지 않습니다

- 공식 Matter 인증 절차는 별도로 처리됩니다

- 인증 번들 플래싱 기능은 향후 전용 로컬 도구를 통해 지원될 예정입니다

2. 하드웨어 정책 (Hardware Policy)

Reference Board

ESP32-C6 DevKitC-1

입력 GPIO

- 0

- 1

- 2

- 3

출력 GPIO

- 18

- 19

- 20

- 21

- 22

- 23

보드 가이드

- 왼쪽 = 입력

- 오른쪽 = 출력

이 범위를 벗어나는 GPIO는

- 서버 정책

- 펌웨어 정책

모두에서 유효하지 않은 값으로 처리됩니다.

3. 레이어 분리 정책 (Layer Separation Policy)

3-1. Firmware Policy Layer

다음 항목들은 Anna JSON 외부에서 정의됩니다.

- DataModelRevision

- SpecificationVersion

- CapabilityMinima

- MaxPathsPerInvoke

- CommissioningFlow

- DiscoveryCapabilities

- FirmwareHash

3-2. Product Layer

ProductInfo에서 관리됩니다.

- VID / PID

- vendor / product naming

- HW / SW 버전

- 사용자 표시 라벨

- 인증 상태

3-3. Unit Layer

UnitInfo에서 관리됩니다.

- SerialNumber

- UniqueID

- Setup Discriminator

- Setup PIN / Verifier

3-4. Action Layer

Anna JSON Action Definition에서 관리됩니다.

- Button

- Switch

- Modes

- ConButton

- ConSwitch

3-5. 구현 제한 사항

현재 참조 구현은 다음을 지원합니다.

• 최대 5개의 버튼/ConButton 인스턴스

• 최대 3개의 스위치/ConSwitch 인스턴스

• 장치당 최대 6개의 액션 인스턴스

• 장치당 하나의 모드 세트.

4. 식별자 정책 (Identifier Policy)

4-1. SerialNumber

외부에 노출되는 식별자

예시 형식

YYMMDD + XXXXXX

예

250717000001

4-2. UniqueID

내부 전용 고유 식별자

특징

- ULID 기반 형식 사용

- QR 코드 또는 사용자 UI에 노출되지 않아야 함

- 디바이스 NVS 또는 서버 DB에 저장될 수 있음

4-3. UserID

사용자 계정 식별자는 Anna JSON 명세 범위에 포함되지 않습니다.

이는 플랫폼 / 서버 정책에 속합니다.

5. Commissioning 정책

다음 항목들은 Anna JSON 스키마 필드가 아니라 디바이스 프로비저닝 데이터입니다.

- Setup PIN Code

- Discriminator

- SPAKE2+ verifier

- QR data

현재 정책

- SetupPINCode / Discriminator는
ESP-Matter HRNG 자동 생성을 사용합니다.

정책 유형

- fixed
Factory Reset 이후에도 유지

- dynamic
Factory Reset 이후 재생성 후 다시 저장

6. Factory Reset 정책

Trigger

다음 조건 중 하나

- 약 10초 내에 전원 ON/OFF 5회 반복

- ScheduleFactoryReset() 정책 호출

- 실제 동작은 펌웨어 브랜치에 따라 다를 수 있음

Effects

Factory Reset은 런타임 상태와 앱 관리 상태를 초기화합니다.

하지만 Anna JSON 자체를 자동으로 재정의하지는 않습니다.

삭제되는 항목

- app 관리 NVS 데이터

- 사용자 설정

- 네트워크 자격 정보

- 런타임 상태

- commissioning / fabric 상태

유지 또는 재생성되는 항목

- firmware 유지

- SerialNumber 유지

- UniqueID 유지

- Setup PIN / Discriminator는 fixed/dynamic 정책에 따름

7. Apply Validation 정책

새로운 Anna JSON payload를 적용하기 전에
시스템은 반드시 다음을 검증해야 합니다.

- output pin 충돌

- 보호 GPIO 규칙

- mode 일관성

- 허용된 GPIO 범위

- payload 무결성

유효하지 않은 payload는 런타임 적용 전에 반드시 거부해야 합니다.

8. Mode Runtime 정책

- Mode는 자유로운 라벨이 아니라 runtime / cluster 정책으로 구현됩니다

- 동시에 하나의 Mode만 활성화될 수 있습니다

- 마지막 활성 Mode는 NVS에 저장됩니다

- 재부팅 시 저장된 Mode가 복원됩니다

다음 경우 예외 처리

- 저장된 Mode가 유효하지 않거나

- NotModePin 조건에 의해 차단된 경우

→ 시스템은 mode index 0으로 fallback

또한

Mode Off가 발생하면

→ mode index 0으로 복귀

9. ON/OFF 상태 복구 정책

StartUpOnOff 복구는

Matter StartUpOnOff = 2

를 기본 기준으로 사용합니다.

그러나 최종 복구 상태는 다음 조건을 추가로 고려합니다.

- protection GPIO 조건

- 저장된 상태 무결성

- 초기화 안전 정책

이 검증이 실패하면

최종 상태는 안전 OFF 상태로 보정될 수 있습니다.

10. Update Path 정책

개발 / 인증 이전 디바이스

다음 runtime 업데이트 경로가 허용될 수 있습니다.

- local IP / HTTPS

- vendor cluster path

- OTA 기반 config 전달

- push 알림 후 pull

인증 릴리스 디바이스

인증된 릴리스 디바이스는

신뢰된 로컬 플래싱 절차만 허용해야 합니다.

런타임 서버 측 설정 주입은

release 모드에서 비활성화되어야 합니다.

11. Certification 정책

릴리스 디바이스 조건

- CSA 발급 VID / PID

- 공식 DAC / PAI

- CertificationID 기록

- 인증 번들은 신뢰된 로컬 도구로 적용

- HSM 기반 제조 도구 사용 가능

Anna JSON 자체는

Action 정의 레이어일 뿐이며 인증 자산을 포함하지 않습니다.

12. Firmware Constant 정책

현재 reference profile의 펌웨어 상수

DataModelRevision = 1
SpecificationVersion = 0x01040100
CapabilityMinima:
  SubscriptionsPerClient = 4
  Fabrics = 3
MaxPathsPerInvoke = 2
CommissioningFlow = 0
DiscoveryCapabilities = 0x03

FirmwareHash

- firmware.bin SHA-256 기반 생성

- read-only firmware constant로 삽입

FirmwareHash는

- 펌웨어 무결성에만 적용

다음에는 적용되지 않습니다.

- ProductInfo

- Anna JSON 내용

13. Release Separation 정책

인증 릴리스 디바이스는

단일 플래그가 아니라 조건 세트를 만족해야 합니다.

권장 검증 항목

- release VID / PID

- 공식 DAC / PAI

- release firmware profile

- release signing chain

이 중 하나라도 불일치하면

- release 경로 거부

- 또는 safe fallback 처리

14. 문서 경계 (Document Boundary)

이 운영 문서는 다음을 포함합니다.

- commissioning 정책

- reset 정책

- update 경로 정책

- firmware constant 정책

- certification 분리 정책

- runtime validation 규칙

이 문서는 다음을 포함하지 않습니다.

- Anna JSON 필드별 스키마 정의

- FunctionType 필드 정의

- 필드 타입 테이블

이 내용은 다음 문서에 속합니다.

Anna JSON FunctionType Specification