Anna JSON Specification

(Action-Centric Device Definition Model for Matter-Based IoT Platforms)

Version: v1.2
Status: Public Draft
Authors: AnnaSoft Inc. Nemo & Anna Platform Team
Last Updated: 2026-03-27
License: Creative Commons Attribution 4.0 International (CC BY 4.0)

License Summary

You are free to:

- share — copy and redistribute the material in any medium or format

- adapt — remix, transform, and build upon the material

for any purpose, even commercially, under the condition that appropriate credit is given.

0. Purpose

Anna JSON is the configuration language used by the Nemo & Anna platform
to declaratively define device actions, boundaries, and execution constraints.

- Anna JSON is not a sensor-first description. It is an action-centered declaration of what a device is allowed to do, under which conditions, and within which boundaries.

- Anna JSON does not merely describe how a device operates. It defines the conditions, limits, and responsibility boundaries that must be checked before execution.

- In Anna JSON, every executable action is reduced to one of two primitives: Button or Switch.

- Mode is not an extra feature layer. It is a constraint layer that determines when a declared action is valid.

- The goal of Anna JSON is not unlimited automation, but executable clarity: every action should be understandable, reviewable, and bounded before it runs.

1. Scope

Anna JSON defines executable behavior only.

The following concerns are managed in separate layers:

- UnitInfo: per-device identity

- ProductInfo: per-product identity and user-visible properties

- Anna Action Definition: executable actions and mode constraints

Firmware policy may be managed outside Anna JSON,
and is not versioned through Anna JSON schemaVersion fields.

2. Transmission Envelope

Anna JSON is transmitted as:

{ "meta": { ... }, "data": { ... } }

- meta: transport and validation metadata

- data: actual payload content

2.1. Meta fields

- schemaVersion

- payloadByteLength

- crc32c

2.2. Encoding

Payload encoding is fixed to UTF-8 by specification and is not repeated in each payload object.

2.3. Schema Version Model (Revised)

Anna JSON uses a dual-level versioning model:

- Global Version (meta)

- Local Version (block-level, optional)

Global Version (meta)

The meta.schemaVersion defines how the entire payload MUST be interpreted.

Rules:

- MUST be present in every payload

- MUST be the single authoritative version for payload parsing

- All receivers MUST rely on this value to interpret the payload

Local Version (block-level)

Individual blocks MAY define their own version for structural evolution.

Example:

"ProductInfo": {
  "ProductInfoVersion": "AnnaProduct.v1"
}

Rules:

- Block-level version fields are OPTIONAL

- They define only the structure of the specific block

- They MUST NOT override or affect payload-level interpretation

Design Principle

This model ensures:

- consistent payload interpretation (via Global Version)

- flexible evolution of internal structures (via Local Version)

Summary

- meta.schemaVersion → Global, authoritative

- ProductInfoVersion → Local, structural only

3. Hardware Reference Board Policy

Current reference board: ESP32-C6 DevKitC-1

- Input GPIO: 0–3

- Output GPIO: 18–23

All PinNo, NotOnPin, NotOffPin, and NotModePin values must remain within this policy.

PinNo must be unique across all output actions
(Button, Switch, ConButton, ConSwitch).

Implementation limits are defined in Operational Policy.

4. FunctionType Summary

| FunctionType | Name | Structure | Description |
| --- | --- | --- | --- |
| 0 | UnitInfo | Object | Per-device identity |
| 1 | ProductInfo | Object | Product-level identity |
| 11 | BoolSens | Array | Reserved digital sensor |
| 21 | AdcSens | Array | Reserved analog sensor |
| 31 | I2CSens | Array | Reserved I2C sensor |
| 41 | SetTarget | Array | Reserved target-setting function |
| 51 | Button | Array | Momentary pulse output |
| 61 | Switch | Array | Maintained output |
| 71 | Modes | Object | Mode constraint definition |
| 81 | ConButton | Array | Conditional momentary output |
| 91 | ConSwitch | Array | Conditional maintained output |

5. Common Rules

5.1. Validation before apply

Before applying an Anna JSON definition, the device MUST validate:

- pin conflicts

- protection GPIO constraints

- mode consistency

Invalid definitions MUST be rejected.

5.2. ActionLabel and mode label uniqueness

Within a single payload:

- Action labels (ActionLabel) MUST be unique

- Mode labels MUST be unique

5.3. ActionLabel semantics

In Anna JSON, the Label field of every executable action
(Button, Switch, ConButton, ConSwitch) is interpreted as the ActionLabel.

The ActionLabel defines the canonical human-readable identity of the action.

It is not decorative. It represents what the action is called in human terms.

Rules

- The Label field MUST appear exactly once per action object

- The value MUST be a human-readable string

- The Label MUST be unique across all actions

- The Label MUST represent the primary identity of the action

- The Label MUST NOT represent:

  - hardware position (Left, Right)

  - orientation (Top, Bottom)

  - endpoint index or numbering

  - manufacturer-internal codes

- The Label SHOULD remain stable across reboots and updates

No other field in Anna JSON may redefine or replace this identity.

Platforms MAY use this value as the preferred name unless overridden by the user.

5.4. FixedLabel policy

- Up to 5 entries per action by default

- Expandable by platform policy

- Label/Value length: 2–32 characters

Allowed characters:

a-z A-Z 0-9 space _ < > = ! , : / +

FixedLabel is used for:

- boundaries

- warnings

- intended use

- non-user-editable metadata

FixedLabel MUST NOT define or override action identity.

5.5. Input slot interpretation

Input GPIO 0–3 are shared logical slots:

- BSPinNo* → digital context

- ASPinNo* → analog context

6. FuncType 0 — UnitInfo

Per-device identity object.

Fields

| Field | Type | Description |
| --- | --- | --- |
| SerialNumber | string | External-facing unit serial number |
| UniqueID | string | Internal unique identifier, ULID-based |

Rules

- UniqueID MUST be ULID-based

- UniqueID is internal-only

- SerialNumber may be exposed

Example

"UnitInfo": {
  "SerialNumber": "250717000001",
  "UniqueID": "01J9V8M6YQ4Q0Y8T7J6A2B3C4D"
}

7. FuncType 1 — ProductInfo

Product-level identity and user-visible properties.

Fields

| Field | Type | Description |
| --- | --- | --- |
| ProductInfoVersion | string | Content schema version for ProductInfo |
| VendorID | uint16 | Matter Vendor ID |
| VendorName | string | Vendor name |
| ProductID | uint16 | Product ID |
| ProductName | string | Product name |
| NodeLabel | string | User-visible device name |
| ProductLabel | string/null | Optional product label |
| Location | string/null | Installation location |
| HardwareVersion | uint16 | Hardware version |
| HardwareVersionString | string | Hardware version string |
| SoftwareVersion | uint32 | Numeric software version |
| SoftwareVersionString | string | Software version string |
| isCertified | bool | Matter certification status |

ProductInfoVersion is a block-level version field as defined in Section 2.3.

Example

"ProductInfo": {
  "ProductInfoVersion": "AnnaProduct.v1",
  "VendorID": 65524,
  "VendorName": "MyCompany",
  "ProductID": 32799,
  "ProductName": "Fruit Box",
  "NodeLabel": "Fruit Box",
  "ProductLabel": "Fruit Box",
  "Location": "Living Room",
  "HardwareVersion": 1,
  "HardwareVersionString": "1.0.0",
  "SoftwareVersion": 100,
  "SoftwareVersionString": "1.0.0",
  "isCertified": false
}

8. FuncType 51 — Button

Momentary output action.

- Executes once and terminates

- No state maintained

Rules

- No NotOffPin

- ActionLabel must be unique

- PinNo must be unique among active output actions

- Executes as a single pulse and terminates automatically

- Does not maintain state

Fields

| Field | Type | Description |
| --- | --- | --- |
| Label | string | ActionLabel (primary human-readable identity of the action) |
| PinNo | int | Output GPIO |
| FixedLabel | array/null | Declared boundaries |
| NotOnPin | array/null | GPIO conditions blocking ON |

Example

"Button": [
  {
    "Label": "Orange Out",
    "PinNo": 18,
    "FixedLabel": [
      { "Label": "limit", "Value": "max-daily-3x" },
      { "Label": "warning", "Value": "thermal-risk" },
      { "Label": "intended-use", "Value": "attended" }
    ],
    "NotOnPin": [20, 22, 23]
  }
]

9. FuncType 61 — Switch

Maintained output action.

- Holds ON/OFF state

Rules

- ActionLabel must be unique

- PinNo must be unique among active output actions

- Maintains ON or OFF state

- State transitions may be blocked by protection conditions

Fields

| Field | Type | Description |
| --- | --- | --- |
| Label | string | ActionLabel (primary human-readable identity of the action) |
| PinNo | int | Output GPIO |
| FixedLabel | array/null | Declared boundaries |
| NotOnPin | array/null | GPIO conditions blocking ON |
| NotOffPin | array/null | GPIO conditions blocking OFF |

Example

"Switch": [
  {
    "Label": "Apple Out",
    "PinNo": 19,
    "FixedLabel": [
      { "Label": "limit", "Value": "max-daily-3x" },
      { "Label": "warning", "Value": "thermal-risk" },
      { "Label": "intended-use", "Value": "attended" }
    ],
    "NotOnPin": [20, 23],
    "NotOffPin": [21, 22]
  }
]

10. FuncType 71 — Modes

Constraint layer for executable actions.

Rules

- ModeNo means total number of available modes

- Valid mode indexes are always: 0 ~ ModeNo-1

- Allowed ModeNo: 2–4

- Mode labels must be unique

- Mode may be null in conditional actions if no mode restriction is required

Fields

| Field | Type | Description |
| --- | --- | --- |
| ModeNo | int | Total number of modes (2–4) |
| Label | array/null | Ordered list of mode labels (primary human-readable identity) |
| NotModePin | array/null | GPIO conditions blocking mode transition |

Example

"Modes": {
  "ModeNo": 3,
  "Label": ["Orange Mode", "Apple Mode", "Mango Mode"],
  "NotModePin": [20, 21, 23]
}

In this example, valid mode indexes are: 0, 1, 2

Runtime behavior of Modes is defined in Operational Policy.

11. FuncType 81 — ConButton

Conditional momentary output action.

Rules

- No NotOffPin

- Same pulse behavior as Button

- MaxSec means responsibility limit, not pulse width

Fields

| Field | Type | Description |
| --- | --- | --- |
| Label | string | ActionLabel (primary human-readable identity of the action) |
| PinNo | int | Output GPIO |
| FixedLabel | array/null | Declared boundaries |
| Mode | int/null | Required mode index |
| BSPinNo1~2 | int/null | Digital input pin |
| BSEvent1~2 | bool/null | Expected digital state |
| ASPinNo1~2 | int/null | Analog input pin |
| TarValue1~2 | int/null | Target value (1–99) |
| ASEvent1~2 | bool/null | true=upper bound, false=lower bound |
| MaxSec | int/null | Responsibility limit in seconds |
| NotOnPin | array/null | GPIO conditions blocking ON |

Example

"ConButton": [
  {
    "Label": "Apple Pulse",
    "PinNo": 20,
    "FixedLabel": [
      { "Label": "limit", "Value": "max-daily-3x" },
      { "Label": "warning", "Value": "thermal-risk" },
      { "Label": "intended-use", "Value": "attended" }
    ],
    "Mode": 1,
    "BSPinNo1": 0,
    "BSEvent1": true,
    "BSPinNo2": 1,
    "BSEvent2": false,
    "ASPinNo1": 2,
    "TarValue1": 25,
    "ASEvent1": true,
    "ASPinNo2": null,
    "TarValue2": null,
    "ASEvent2": null,
    "MaxSec": 20,
    "NotOnPin": [23]
  }
]

12. FuncType 91 — ConSwitch

Conditional maintained output action.

Rules

- Same structure as ConButton

- No MaxSec

- Maintains state until OFF command or runtime policy change

Fields

| Field | Type | Description |
| --- | --- | --- |
| Label | string | ActionLabel (primary human-readable identity of the action) |
| PinNo | int | Output GPIO |
| FixedLabel | array/null | Declared boundaries |
| Mode | int/null | Required mode index |
| BSPinNo1~2 | int/null | Digital input pin |
| BSEvent1~2 | bool/null | Expected digital state |
| ASPinNo1~2 | int/null | Analog input pin |
| TarValue1~2 | int/null | Target value (1–99) |
| ASEvent1~2 | bool/null | true=upper bound, false=lower bound |
| NotOnPin | array/null | GPIO conditions blocking ON |
| NotOffPin | array/null | GPIO conditions blocking OFF |

Example

"ConSwitch": [
  {
    "Label": "Mango Out",
    "PinNo": 22,
    "FixedLabel": [
      { "Label": "limit", "Value": "max-daily-3x" },
      { "Label": "warning", "Value": "thermal-risk" },
      { "Label": "intended-use", "Value": "attended" }
    ],
    "Mode": 2,
    "BSPinNo1": 0,
    "BSEvent1": false,
    "BSPinNo2": 1,
    "BSEvent2": true,
    "ASPinNo1": 3,
    "TarValue1": 25,
    "ASEvent1": false,
    "ASPinNo2": null,
    "TarValue2": null,
    "ASEvent2": null,
    "NotOnPin": [20, 23],
    "NotOffPin": [21]
  }
]

13. Example A — UnitInfo Payload

{
  "meta": {
    "schemaVersion": "AnnaUnit.v1",
    "payloadByteLength": 121,
    "crc32c": "93002FA0"
  },
  "data": {
    "UnitInfo": {
      "SerialNumber": "2507170001",
      "UniqueID": "01J9V8M6YQ4Q0Y8T7J6A2B3C4D"
    }
  }
}

14. Example B — Full Anna Device Definition Payload

{

"meta": {

"schemaVersion": "AnnaJson.v1",

"payloadByteLength": 0,

"crc32c": "TBD"

},

"data": {

"ProductInfo": {

"ProductInfoVersion": "AnnaProduct.v1",

"VendorID": 65524,

"VendorName": "MyCompany",

"ProductID": 32799,

"ProductName": "Fruit Box",

"NodeLabel": "Fruit Box",

"ProductLabel": "Fruit Box",

"Location": "Living Room",

"HardwareVersion": 1,

"HardwareVersionString": "1.0.0",

"SoftwareVersion": 100,

"SoftwareVersionString": "1.0.0",

"isCertified": false

},

"Modes": {

"ModeNo": 2,

"Label": ["Coffee Mode", "Standby Mode"],

"NotModePin": [20]

},

"Button": [

{

"Label": "Start Brewing",

"PinNo": 18,

"FixedLabel": [

{ "Label": "limit", "Value": "max-daily-5x" },

{ "Label": "warning", "Value": "hot-surface" }

],

"NotOnPin": [22]

}

],

"Switch": [

{

"Label": "Coffee Heater",

"PinNo": 19,

"FixedLabel": [

{ "Label": "limit", "Value": "max-10min" },

{ "Label": "warning", "Value": "thermal-risk" }

],

"NotOnPin": [20],

"NotOffPin": [21]

}

],

"ConButton": [

{

"Label": "Auto Clean Pulse",

"PinNo": 20,

"FixedLabel": [

{ "Label": "intended-use", "Value": "maintenance" }

],

"Mode": 0,

"BSPinNo1": 0,

"BSEvent1": true,

"ASPinNo1": 2,

"TarValue1": 30,

"ASEvent1": true,

"MaxSec": 10,

"NotOnPin": [23]

}

],

"ConSwitch": [

{

"Label": "Keep Warm",

"PinNo": 21,

"FixedLabel": [

{ "Label": "limit", "Value": "max-30min" },

{ "Label": "warning", "Value": "thermal-risk" }

],

"Mode": 0,

"BSPinNo1": 1,

"BSEvent1": true,

"ASPinNo1": 3,

"TarValue1": 40,

"ASEvent1": false,

"NotOnPin": [22],

"NotOffPin": [23]

}

]

}

}

15. Summary

- All payloads use a fixed { meta, data } structure

- Null is allowed where explicitly defined

- Reserved FunctionTypes are kept for future expansion

- Button and Switch are the two action primitives

- Modes define validity constraints, not extra features

- Anna JSON changes must be treated as versioned configuration changes

- The Label field of every executable action is interpreted as ActionLabel, the canonical human-readable identity of that action.

16. Matter Interoperability

Exactly one UserLabel entry is transmitted per endpoint to ensure a fixed identity.

16.1 Matter TLV Encoding (Conceptual)

Matter TLV (Conceptual Encoding of UserLabel)

UserLabelList = Anonymous Array [

Structure {

ContextTag(0): "endpointLabel"

ContextTag(1): "<endpoint UI name>"

}

]

The value of `endpointLabel` is the UI-visible name of the endpoint.

FixedLabel MAY include multiple entries using the same TLV encoding structure.